PROJECT PIPELINE PROPOSAL — LUCA CREATINI, UNIVERSITY OF BOLOGNA ALMA MATER

How Often Would a Satellite Have Warned You?

A retrospective space-time coincidence and re-routing added value for SAR- and altimeter-detected hazardous sea states along North Atlantic shipping corridors

BASE PAPER & DISCIPLINARY ORIENTATION Base paper: Kytariolou, A.; Themelis, N. Ship routing optimisation based on forecasted weather data and considering safety criteria. Journal of Navigation 2022, 75(6), 1310–1331. Disciplinary orientation: This project takes [1] as its empirical starting point, but is reoriented toward physical oceanography (extreme waves generation and detectability at sub-forecast-grid scales) and satellite oceanography (SAR and altimeter sampling geometry).

# 1. Objective

The aim of the project is to build and test a retrospective framework that, for a realistic North Atlantic containership route (like in [1], Lisbon-Halifax), measures:

- How often and where a satellite pass would actually have coincided in space-time with a hazardous sea-state event that the deterministic weather-routing forecast underrepresented

- What operational value (expressed as safety margin recovered against fuel-cost penalty incurred) a hypothetical satellite-informed re-routing would have added to [1]’s own fuel/safety optimization framework.

The final deliverable is a parametric Added Detection Value (ADV) atlas along the corridor.

# 2. Rationale

The routing tool built in [1] optimizes fuel consumption subject to fixed seakeeping safety thresholds (maximum significant heave and roll) entirely evaluated on deterministic CMEMS forecast fields updated every 3-12 hours. The tool’s conclusions treat forecast accuracy as an assumed input, never cross-checked against independent observations. This is physically relevant because extreme waves (“rogue waves”), which are the events that most likely will breach a safety threshold all of a sudden, have horizontal scales of hundreds-of-meters to few-kilometers and durations of minutes, well below what a 3-hourly degree-scale forecast grid can resolve.

Since MaxWave Project [2], C-band SAR imagery has been shown capable to image such individual extreme events directly and independently of any forecast model, an ability recently further developed with cross-track interferometric SAR [3]. On a parallel and much longer standing track, satellite altimetry has been validated for decades as an independent significant-wave-height observation, usually used for wave-model bias correction [4, 5].

What has never been quantified, as far as the author knows, is a simple operational question that determines whether any of this actually matters for a specific route and vessel (like in [1]): given real satellite orbital geometry and revisit patterns, how often would a satellite actually have been overhead at the moment a hazardous sea state occurred and the forecast missed it? And on those occasions how much would that information have been worth inside the exact fuel/safety trade-off framework [1] already built?

This project aims to answer these questions retrospectively, on entirely open data, by treating satellite detectability as a space-time sampling and coincidence problem: the interaction between two independently operating systems, a shipping route and a satellite constellation.

# 3. Hypotheses

H1 — Space-time coincidence probability between a hazardous sea-state event (Criteria in Phase 1) and a contemporaneous Sentinel-1 acquisition footprint over the corresponding route segment is low for a single SAR mission/mode, but increases substantially when the full open-data SAR constellation is combined with complementary altimeter tracks.

H2 — Coincidence probability is spatially heterogeneous along Lisbon-Halifax corridor: it is notably higher near Grand Banks / Flemish Cap approach (dense SAR tasking for fisheries and ice monitoring) than along the mid-ocean leg, producing an open Atlantic “blind corridor” where satellite-informed re-routing could not plausibly help regardless of the mechanism’s physical relevance.

H3 — For the subset of coincident events, CMEMS deterministic forecast used in [1]’s rooting tool significantly underestimates the peak sea-state relative to the independent altimeter/SAR-derived observation, consistent with forecast’s 3-hourly resolution being unable to resolve short-lived extreme-wave events.

H4 — Reintroducing satellite-observed hazard as an explicit avoidance constraint into a retrospective re-run of a [1]-style route optimization recovers compliance with the original safety thresholds at a quantifiable and modest fuel-cost penalty, establishing (where available) that the operational value of satellite information exceeds its cost within the routing framework [1] already built.

# 4. Study Area and Data

4.1 Route and Vessel

The project reuses [1]'s Lisbon–Halifax route and the containership described in its Table 1 (LBP 244.80 m, 4250 TEU, DWT 50,829 t, MAN B&W 8K90MC-C main engine, service speed 24.5 kn), so that the fuel/safety framework reproduced in Phase 1 is directly comparable to [1]'s published baseline (its Table 2). The route crosses two operationally distinct legs: an open mid-Atlantic leg, where SAR coverage is expected to be sparse, and the Grand Banks/Flemish Cap approach, historically a zone of dense SAR/RCM tasking for iceberg and fisheries surveillance and a major extratropical cyclogenesis corridor, giving H2 a genuine physical and operational contrast to test.

  - Data Sources and Instrumentation

— Ocean waves (hazard timeline): CMEMS Global Ocean Waves Reanalysis (GLOBAL_MULTIYEAR_WAV_001_032)  significant wave height, mean/peak direction, peak period, ~3h resolution, 1980–present.

— Ocean physics (currents, for fuel-model consistency with [1]): CMEMS Global Ocean Physics Reanalysis (GLOBAL_MULTIYEAR_PHY_001_030)  currents, temperature, salinity, ~1/12°, 1993–present.

— Wind: ERA5 (Copernicus Climate Data Store), 1940–present, cross-checked against the wind forcing embedded in the CMEMS wave reanalysis.

— Independent satellite Hs (observational cross-check, H3): CMEMS Global Ocean L3 significant wave height from reprocessed multi-mission altimetry (WAVE_GLO_PHY_SWH_L3_MY_014_005), along-track, 1991–present; L4 merged gridded product (WAVE_GLO_PHY_SWH_L4_MY_014_007) as a secondary cross-check.

— SAR acquisition metadata (coincidence analysis, H1/H2): Copernicus Data Space Ecosystem STAC/OData catalogue  footprints and acquisition times for Sentinel-1 IW/EW/WV modes, 2014–present, with continuity from Sentinel-1C; cross-checked against the openly searchable Alaska Satellite Facility (ASF) Vertex archive.

— SAR-derived wave observations (case-study validation subset): Sentinel-1 Level-2 Ocean (OCN) Ocean Swell Wave spectra (OSW) product [4], freely distributed alongside Level-1 SAR data, applied to a limited number of coincident events identified in Phase 3 (not to the full multi-year dataset).

— Ship / fuel / seakeeping baseline: vessel characteristics and safety thresholds (maximum significant heave 3.5 m, maximum significant roll 5°) published in [1] (Table 1, Sec. 4.6), reproduced here.

— Hazard-definition anchor: abnormality-index criterion (Hmax/Hs) for individual-wave exceedance, following the documented Draupner event characterization [6].

4.3 Access Constraints

Every dataset above is open-access and queryable through public APIs (Copernicus Marine Toolbox, Copernicus Data Space Ecosystem API, CDS API for ERA5), with no registration fee and no data-sharing agreement. It is a stronger zero-cost position rather than [1] itself, whose seakeeping computations depended on a non-public code (NEWDRIFT+). Because that engine will not be reproduced here, Phase 1 substitutes it with an openly available strip-theory tool (e.g. SEAWAY, TU Delft, distributed for academic use) or, if that proves inaccessible within the project's zero-cost constraint, with published parametric roll/heave operability limits for comparable ship hull forms (if so, documented explicitly as a limitation (Sec. 8)).

# 5. Methodology — Pipeline

Phase 0 — Scoping: Copernicus and Satellites

–  Free registration for Copernicus Marine Service, Copernicus Data Space Ecosystem and Copernicus Climate Data Store APIs. Installation and test of Copernicus Marine Toolbox, a STAC client (pystac-client) and CDS API on a one-month trial subset over the Grand Banks leg.

–  Verification of temporal depth and swath-metadata completeness of Sentinel-1 catalogue for full study period. Verification of SEAWAY’s availability and license terms for relevant hull-form class.

–  Output: ready development environment.

Phase 1 — Coincidence, Hazard and Safety Model

–  Definition of the space-time window (Δt, Δx) that constitutes a “coincidence” in Phase 3, based on typical SAR footprint dimensions and hazard event expected extent/duration.

–  Reimplementation of equations 1-18 from [1] of calm-water/wind/wave resistance and fuel consumption, creating a Python module.

–  Reproduction of ship motion (heave and roll) response via open strip-theory tool, qualitatively cross-checking where possible against acceleration curves in [1] Fig 10.

–  Definition of the “hazardous event” flag, combining [1]’s original threshold (significant heave > 3.5 m, significant roll > 5°) and an individual-wave abnormality-index criterion (H_max/H_s > 2, like [6]) for events too short-lived to register in the significant-value criterion.

–  Output: tested Python module reproducing [1]’s fuel/safety framework extended with abnormality index.

Phase 2 — Historical Hazard Timeline

–  Extraction of multi-year (possibly 10 years, depending on Phase 0 catalogue depth check) time series of H_s, wind and currents along Lisbon-Halifax route from CMEMS reanalysis products: both the orthodrome and a small ensemble of candidate routes (mimicking [1]’s route generation logic).

–  Application of Phase 1 hazard flag to the full time series, producing a geolocated timestamped catalogue of candidate hazardous transits.

–  Output: multi-year route-resolved hazard event catalogue.

Phase 3 — Satellite Space-Time Coincidence Analysis

–  For every cataloged hazard event, query of Sentinel-1 acquisition catalogue (all modes) and of altimeter L3 track catalogue for footprints intersecting the event’s space-time window (Phase 1).

–  Computation of coincidence probability statistics as a function of route segment, mission/mode and season (H1, H2).

–  For coincident events with an available altimeter track or Sentinel-1 OSW product, direct comparison of observed H_s (altimeter) and H_max (OSW) against CMEMS forecast-equivalent value used in Phase 1, quantifying forecast underestimation (H3).

–  Output: coincidence probability atlas (preliminary) and quantified forecast-bias statistics for coincidence event subset.

Phase 4 — Re-Routing Added Value Simulation

–  For the “coincidence and forecast-underestimated” events subset (H3), retrospective re-run of a simplified route/speed optimization step (mimicking [1]’s genetic algorithm) with satellite-observed hazard imposed as an explicit avoidance/speed-reduction constraint at the relevant waypoint.

–  Quantification of the resulting change in significant heave/roll (recovered compliance with [1]’s thresholds) against fuel consumption penalty incurred, directly comparable in units to [1] Table 2.

–  Output: paired “safety-margin recovered VS fuel penalty incurred” statistics for each simulated event and aggregate cost/benefit distribution.

Phase 5 — Synthesis and Products

–  Construction of Added Detection Value (ADV) atlas: a route-resolved map combining coincidence probability (Phase 3), forecast-bias magnitude (Phase 3) and re-routing cost/benefit (Phase 4), identifying where a satellite informed nowcast would realistically add operational value to a [1]-style routing tool and where it structurally cannot (H2 blind corridor)

–  Discussion of latency/coverage conditions (mission, mode, revisit) under which added value becomes operationally actionable, as opposed to purely diagnostic and retrospective.

– 	Release of the Phase 1 hazard/fuel module and Phase 3 coincidence analysis code as reusable open-source package.

–  Output: ADV atlas, Manuscript and Code.

# 6. Pipeline Overview

| Phase | Focus | Key features | Output |
| --- | --- | --- | --- |
| 0 | Scoping | Tools Setup | Data and Computational Environment |
| 1 | Coincidence, Hazard/Safety Model | Equations Implementation – Strip-theory RAO – Abnormality Index | Python Module |
| 2 | Hazard Timeline | CMEMS data – hazard flagging | Hazard Event Catalogue |
| 3 | Satellite Coincidence | Catalogues query – Coincidence stats – Bias check | Coincidence Atlas and Bias Statistics |
| 4 | Re-Routing Value | Retrospective Optimization – Safety vs Fuel | Cost/Benefits Statistics |
| 5 | Synthesis / Products | ADV atlas – Operational discussion | ADV Atlas, Manuscript and Open-Source Code |

# 7. Expected Outputs

–  An Added Detection Value (ADV) atlas quantifying, for the first time as far as the author knows, where along a real North Atlantic shipping corridor satellite-informed nowcasting would add measurable operational value to a deterministic forecast routing tool like [1] and where it structurally cannot (“blind corridors”).

– 	An open-source Python package combining a reproducible fuel/safety module (extending [1]) with a general-purpose satellite space-time coincidence analyzer, reusable for any route/mission combination beyond this project.

–  A quantitative route-specific statement on CMEMS wave-forecast underestimation during hazardous satellite-coincident events.

–  A manuscript.

# 8. Limitations

The project does not reproduce [1]’s proprietary seakeeping engine (NEWDRIFT+): Phase 1 substitutes an open strip-theory tool or (in the worst scenario) published parametric operability limits, which introduces uncertainty into exact heave/roll thresholds not present in [1] original result. It should be treated as sensitivity.

Coincidence (computed here from an archival catalogue of past acquisitions) is not the same thing as guaranteed real-time detection and alerting. An operational system would additionally require low-latency SAR/altimeter products and an automated onboard (or shoreside) alerting chain, both explicitly out of this project scope.

Full-swath pixel-level SAR wave/hazard retrieval is applied only to a limited case-study (Phase 3) to preserve project’s zero-cost. Scaling the image processing step to the full dataset is a natural but heavier extension.

Phase 4 retrospective re-routing is for its nature counterfactual. It cannot verify that a real captain would have acted on the information, nor capture commercial constraints beyond fuel/safety criteria in the model.

The abnormality index criterion (H_max/H_s) applied to a model-grid or altimeter-track H_s value is itself a statistical Rayleigh-based expectation (it is not a proven predictor at the exact point): it is used as a hazard-likelihood proxy.

# 9. Modular Extension Tracks Team & Time Scale-Up

The core pipeline above is a complete submittable project on its own. Everything below is organized as independent addable modules: each module plugs into one existing phase and does not alter phase original output, nor require other modules to exist. With a large team (and ambition), as long as the hackathon clock allows, it is possible to implement one or more of these modules.

Module A – Second Hazard Channel: Icebergs on the Grand Banks Approach

Plugs into Phase 1 (hazard definition) and Phase 3 (coincidence), as a second hazard branch running in parallel to rogue waves.

Rationale The Grand Banks/Flemish Cap approach leg is already flagged in H2 as a dense-SAR zone and icebergs are the historically dominant hazard there. The International Ice Patrol (US Coast Guard) publishes, freely and continuously since 1913, the seasonal count and southernmost extent of icebergs south of 48°N and daily "limit of all known ice" charts, ready-made open dataset. The exact Phase 3 coincidence machinery is reused unchanged; only a second hazard-flag definition and a second data feed are added.

Data IIP historical iceberg-season records and ice charts (free, US Coast Guard); Canadian Ice Service ice charts (free); SAR ship–iceberg discrimination literature as methodological backing [7].

Owner profile Team member with a polar remote-sensing leaning.

Effort Medium – mostly reuses Phase 1/3 machinery, new work is IIP data ingestion plus a second hazard flag.

Module B – Full Open-Constellation Depth Test

Plugs into Phase 3.

Rationale Strengthen H1. Instead of Sentinel-1 alone, query the complete set of currently operating open sea-state relevant missions (Sentinel-1A/1C SAR; Sentinel-3A/3B and Sentinel-6 Michael Freilich, CryoSat-2, SARAL/AltiKa, HY-2B/C altimetry, already aggregated by source mission in the CMEMS L3 multi-mission product used in Sec. 4.2). Is then possible to compute how coincidence probability scales as missions are added on at a time, producing a novel self-standing “constellation value curve”.

Data No new dataset. Fuller use of the multi-mission catalogue already used in Phase 3.

Owner profile Team satellite data specialist.

Effort Small – parametrization of existing Phase 3 code.

Module C – Open Hydrodynamic Seakeeping Upgrade (NEMOH + KCS Benchmark Hull)

Plugs into Phase 1, replacing SEAWAY (or thresholds fallback), core’s pipeline main scientific credibility gap (Sec 8).

Rationale NEMOH [8] is a fully opensource potential-flow solver able of computing frequency-domain motion RAOs. KRISO Container Ship (KCS) is a documented benchmark hull geometry widely used in seakeeping community, reasonably close to [1]’s vessel. Using it removes the single largest limitation in core pipeline, at the cost of a non-negligible numerical work.

Data NEMOH package, KCS parameters.

Owner profile team member with numerical methods background (possibly naval application).

Effort Large – the most technically demanding module, first to drop if time runs short.

Module D – Live Coincidence Dashboard: Hackathon Demo Layer

Plugs into Phase 5, presentation-layer addition

Rationale Hackathon judgement rewards also interactive artefacts. The same catalogue query logic built for Phase 3 can be pointed at “now” (instead of “historical archive”): a live map showing today’s candidate route, latest CMEMS forecast along it and nearest past/nest satellite pass, with a coincidence indicator. It is a demonstrable proof of concept “in the room”.

Data Now-data from satellites.

Owner profile Best team member in front-end data visualization.

Effort Medium – Visualizing effort, but zero risk to scientific core if cut.

Module E – Economic Framing

Plugs into Phase 5, Sec 7 (outputs).

Rationale Translation of ADV atlas’s fuel/safety numbers into an industry-relevant business case using already published aggregate figures on shipping losses (like [9]), to argue whether acting on satellite-informed hazard information is justified at fleet scale. It is a strengthening of the real-world relevance pitch (without any new primary data needed to collect).

Data Shipping Losses literature [9].

Owner profile Team member with an economics/insurances background.

Effort Small – Writing heavy module, but low technical risk.

Module F – Multi-Corridor Generalization

Plugs into Phase 2.

Rationale Testing whether H2 “blind corridor” pattern generalizes to other North Atlantic (or other basins) corridors by re-running Phase 0-3 machinery on two or three additional representative routes generated with [1] own route-generation logic.

Data New routes to be determined.

Owner profile First team member available after his/her work. Naturally parallelizable.

Effort Medium – Computation time-demanding, but no new development required. Parallelizable, no coordination work required in human cost.

| Mod | Name | Plugs into | Owner profile | Effort | If time runs short |
| --- | --- | --- | --- | --- | --- |
| A | Icebergs (Grand Banks) | Phase 1 & 3 | Polar RS | Medium | Keep |
| B | Full constellation | Phase 3 | Satellite-data specialist | Small | Keep |
| C | NEMOH + KCS seakeeping | Phase 1 | Naval / numerical | Large | Cut first |
| D | Live dashboard | Phase 5 | Data Viz | Medium | Keep |
| E | Economic case | Phase 5 Sec. 7 | Economics / Insurance | Small | Keep |
| F | Multi-corridor | Phase 2 | Any remaining slack | Medium | Cut second |

Suggested idea is to split work for a 5–6 person team. Two people own the core skeleton (Phases 0–3, the MVP that every module depends on and that alone is already a complete submission). One person each takes Modules A, B and D in parallel once the skeleton's Phase-3 coincidence code is stable. Module C is assigned only if a sixth person with the right numerical background is available and is dropped without any knock-on effect on the rest if not. Module E is best scheduled as a shared, writing-heavy task for the final day (depending on the presence of an economics/insurance expert in the team). Module F is the natural filler for whoever finishes early. At every point in this schedule, the team holds a submittable project. The modules only add to it, never gate it.

# 10. References

[1] Kytariolou, A.; Themelis, N. (2022). Ship routing optimisation based on forecasted weather data and considering safety criteria. Journal of Navigation, 75(6), 1310–1331.

[2] Rosenthal, W.; Lehner, S. (2008). Rogue Waves: Results of the MaxWave Project. Journal of Offshore Mechanics and Arctic Engineering, 130(2), 021006.

[3] Wang, T.-C.; Kiang, J.-F. (2025). Detection of Elusive Rogue Wave with Cross-Track Interferometric Synthetic Aperture Radar Imaging Approach. Sensors, 25(9), 2781.

[4] Stopa, J. E.; Mouche, A. (2017). Significant wave heights from Sentinel-1 SAR: Validation and applications. Journal of Geophysical Research: Oceans, 122(3), 1827–1848.

[5] Queffeulou, P. (2004). Long-Term Validation of Wave Height Measurements from Altimeters. Marine Geodesy, 27(3-4), 495–510.

[6] Haver, S. (2004). A possible freak wave event measured at the Draupner Jacket, January 1, 1995. Proceedings of Rogue Waves 2004, Brest, France.

[7] Heiselberg, P.; Sørensen, K. A.; Heiselberg, H.; Andersen, O. B. (2022). SAR Ship–Iceberg Discrimination in Arctic Conditions Using Deep Learning. Remote Sensing, 14(9), 2236.

[8] Babarit, A.; Delhommeau, G. (2015). Theoretical and numerical aspects of the open source BEM solver NEMOH. Proceedings of the 11th European Wave and Tidal Energy Conference (EWTEC 2015), Nantes, France.

[9] Allianz Commercial (Allianz Global Corporate & Specialty). Safety and Shipping Review, published annually (edition to be selected at execution time).

Base paper: ”Ship routing optimisation based on forecasted weather data and considering safety criteria”                                  Disciplinary Framing: Physical and Satellite Oceanography for Marine Services

Email: luca.creatini03@gmail.com

LinkedIn Profile: https://www.linkedin.com/in/luca-creatini-3a574b42b/